# Marco: who builds what, and the interfaces between us (v0.4, 2026-10-18)

Three owners, one contract. Change a message format only by updating this file, the code and tests on both sides, and telling the others. Changes are listed in §11–13.

# 1. Owners

Vedant: perception and world (`server/`, Python). The brain.

- Item identity: registration from one photo plus a name, with the retake check (POC 22, 25, 26).

- Detection at 1 frame/s, with the "is my item in this frame?" decision (POC 25).

- 2D boxes to 3D world positions, using the frame's own pose and depth (POC 23); footprint method for glass.

- Room map: a voxel point cloud built from the depth frames, plus named surfaces (floor, seat, table, counter, shelf, by height).

- World model: items, user position track, and a sightings timeline. Events: seen, put down, moved, picked up, missing, left behind (POC 2 logic).

- Touch detection from the hand stream (POC 24: 3D, fingertip visible only).

- Answers: "where are my keys?" and "what is this?".

- Data feeds: for the phone, Photon and the dashboard.

Utkarsh: the iPhone app (`ios/`, Xcode) and Photon (`photon/`). Sensors in, sound and voice out.

- ARKit session: world tracking, LiDAR scene depth, plane detection; saving and loading the ARWorldMap and relocalizing.

- Sending: frames at 1/s, pose at 10/s, and the hand stream at 15/s while a hand is in view (formats below).

- Spatial audio:

  - AVAudioEnvironmentNode HRTF;

  - AirPods + ARKit head-tracking fusion, with the phone-only fallback;

  - the guidance sequence (§7);

  - label playback.

- Voice: "Marco" plus speech recognition, turned into queries; Marco's spoken lines.

- Photon: sends the caretaker the photo of an unknown item, receives its typed name, and posts it to the server. Voice recordings are not labels.

Dashboard (web, `frontend/`). Reads the server's feeds only (§6):

- the live chest view with boxes;

- the 3D map with items and the user;

- a radar of where the sound is coming from;

- the "what / when / where / last seen" timeline;

- the setup view (items and labels).

# 2. Shared conventions (everyone, no exceptions)

- World frame: ARKit's world coordinates for the current map_id. Metres, +y up (gravity), right-handed.

  - Every position in every message is in this frame.

  - A new map_id means a new coordinate frame, so positions from other maps don't apply.

- Heading and azimuth:

  - heading 0 = world −z, +90 = world +x (turning right);

  - a relative azimuth of 0 means ahead, +90 right, −90 left, ±180 behind;

  - clock positions round half to even, as Python's round does.

- Matrices: send simd_float4x4 / simd_float3x3 as flat float arrays in column-major order, exactly as the columns sit in memory: [c0.x, c0.y, c0.z, c0.w, c1.x, ...].

  - camera_transform = ARFrame.camera.transform (camera to world).

  - intrinsics = ARFrame.camera.intrinsics, for image_resolution = ARFrame.camera.imageResolution, normally 1920×1440.

- Images: the camera image in the sensor's own orientation, which is landscape even though the phone is portrait in the vest.

  - Do not rotate, and do not write an EXIF orientation tag.

  - The server rotates for detection and maps boxes back (POC 23).

- Depth:

  - ARFrame.smoothedSceneDepth.depthMap: 256×192 Float32 metres, sent as float16 little-endian, then DEFLATE-compressed;

  - its confidenceMap: UInt8 0/1/2, DEFLATE-compressed;

  - both in the sensor orientation, row 0 = top of the sensor image.

  - Compression: Apple's NSData.compressed(using: .zlib) writes raw DEFLATE (no zlib header). The server accepts raw DEFLATE or zlib.

- Time:

  - t_device = ARFrame.timestamp, in seconds on the device's uptime clock, for ordering;

  - t_wall = ISO-8601 UTC, for the dashboard.

- Ids:

  - session_id (UUID, per app launch). It is on every phone-to-server message, so the server files everything into one recording.

  - map_id (UUID per ARWorldMap, kept across launches when relocalized);

  - frame_id (increasing per session).

# 3. Phone to server (Utkarsh sends, Vedant receives)

## 3.1 Frames: `POST /v1/frames`, about 1 per second (multipart)

| Part | Content |
| --- | --- |
| meta | JSON, below |
| image | JPEG, 960×720 (the 1920×1440 sensor image scaled 0.5), sensor orientation, quality 0.8, about 100–150 KB |
| depth | DEFLATE(float16 little-endian 256×192) |
| confidence | DEFLATE(uint8 256×192) |

{"session_id": "…", "map_id": "…", "frame_id": 1234, "t_device": 81234.567, "t_wall": "2026-10-17T15:04:05.123Z", "tracking": "normal", "camera_transform": [16 floats], "intrinsics": [9 floats], "image_resolution": [1920, 1440], "jpeg_scale": 0.5}

- Reply: 202 straight away. Detections and events come back over the WebSocket.

- When the server is busy: it keeps only the newest frame from each phone (latest frame wins). The phone does the same: one upload in flight, and only the newest frame waits behind it.

- A `422` reply carries the reason (for example "the JPEG has an EXIF orientation tag", or "JPEG is 720x960 but image_resolution x jpeg_scale is 960x720").

- Don't send frames while tracking isn't normal, or before relocalization finishes (§3.4). The same goes for poses and hands.

## 3.2 WebSocket `wss://…/v1/phone` (JSON, phone to server)

Pose, 10 per second:

{"type": "pose", "session_id": "…", "map_id": "…", "t_device": 81234.6, "camera_transform": [16 floats], "head_yaw_deg": 12.3, "head_source": "fusion"}

head_yaw_deg is the listener yaw the audio engine uses, in the world frame. head_source is fusion or phone.

Hand, 15 per second, only while Vision sees a hand:

{"type": "hand", "session_id": "…", "map_id": "…", "t_device": 81234.62, "camera_transform": [16 floats], "vision_orientation": "up", "chirality": "right", "joints": [{"name": "indexTip", "x": 0.512, "y": 0.431, "conf": 0.88}, … 21 joints, Vision's order], "depth_patches_f16_b64": "…", "conf_patches_u8_b64": "…", "patch": 9}

- Run Vision on `capturedImage` with orientation `.up`, so joint coordinates are in the sensor frame. If accuracy suffers in hour-one tests, switch to .right and tell Vedant; the server will convert.

- Joint order and names: wrist, then thumbCMC/MP/IP/Tip, then index, middle, ring and little, each as MCP/PIP/DIP/Tip. That's Vision's joint names, in server/marco/touch.py JOINTS order.

  - A joint Vision didn't find is sent as x 0, y 0, conf 0.

- Patches: for each joint, in order, a 9×9 window of the depth map (and of the confidence map) centred on depth pixel u = floor(x·256), v = floor((1 − y)·192). Vision's origin is bottom-left; the depth map's is top-left.

  - Pad outside the map with NaN for depth and 0 for confidence.

  - That's 21 × 81 values each, about 5 KB per message.

- Intrinsics: the hand message carries none. The server unprojects with the latest frame's intrinsics, which don't change during a session.

Guidance state, 10 per second while guiding, so the dashboard radar shows what the user hears. Send one last message with phase off when guidance ends:

{"type": "guidance", "session_id": "…", "item_id": "…", "phase": "behind¦turning¦facing¦approach¦reach¦touched¦off", "rel_azimuth_deg": -140, "distance_m": 1.6}

rel_azimuth_deg is relative to the head; distance_m is horizontal, chest to item.

What the wearer hears, for the dashboard (the server only relays these, §6). One sound message the moment each sound starts:

{"type": "sound", "session_id": "…", "t_device": 81235.20, "cue": "beep", "source": "beacon", "anchor": "item", "az_deg": -150.0, "el_deg": -12.0, "distance_m": 1.6, "muffled": true} {"type": "sound", "session_id": "…", "t_device": 81234.90, "cue": "say", "source": "voice", "anchor": "item", "az_deg": -150.0, "el_deg": -12.0, "distance_m": 1.6, "text": "Polo! Behind you, two steps, on the table.", "duration_s": 2.9} {"type": "sound", "session_id": "…", "t_device": 81240.10, "cue": "label", "source": "voice", "anchor": "room", "az_deg": 20.0, "el_deg": -35.0, "distance_m": 0.6, "text": "That's the cumin, love", "duration_s": 1.8}

- `cue`:

  - beep (any beacon pulse or reach ping, with muffled true for the behind sound), ding (facing, and the stop notes), tone_on and tone_off (the steady tone);

  - say (Marco's voice, with text), label (an item's label, spoken in the same voice, with its words in text; audio_url only if it is a recorded voice memo), blip (listening, after "Marco");

  - silence (everything stopped).

- `source`: beacon for beep, ding and tone; voice for say, label and blip.

- `anchor`: item (the guided item), room (another point in the room, such as a touched jar) or ahead (Marco's own lines, fixed to the head).

- Direction, relative to the head at that moment:

  - az_deg: 0 ahead, +90 right, ±180 behind;

  - el_deg: + is up;

  - distance_m: ear to source, in metres;

  - ahead is 0, 0, 1.

- `duration_s`: for say and label, estimated from the words (exact for a voice memo).

Sound field, 10 per second while anything is audible: where each source is now, relative to the head. Send one last message with both sources null when it goes quiet:

{"type": "sound_field", "session_id": "…", "t_device": 81235.30, "head_yaw_deg": 12.3, "beacon": {"az_deg": -150.0, "el_deg": -12.0, "distance_m": 1.6, "muffled": true}, "voice": {"az_deg": 0.0, "el_deg": 0.0, "distance_m": 1.0, "anchor": "ahead"}}

beacon is set while guiding and voice while Marco is speaking; each is null (or absent) otherwise.

Queries:

{"type": "query", "session_id": "…", "query_id": "…", "kind": "find", "name": "keys"} {"type": "query", "session_id": "…", "query_id": "…", "kind": "what_is_this"} {"type": "query", "session_id": "…", "query_id": "…", "kind": "set_place", "name": "door"}

set_place door ("Marco, this is the door") remembers where the user stands. Leaving through there with a carry item (keys, wallet, phone) still in the room triggers left_behind. It needs a pose first.

Map lifecycle:

{"type": "map", "session_id": "…", "event": "relocalized¦new_map¦saved", "map_id": "…"}

Errors: a message the server can't use gets {"type": "error", "detail": "…", "for_type": "hand"} back. The socket stays open.

Command acknowledgement (the answer to a dashboard command, §4):

{"type": "command_ack", "session_id": "…", "command_id": "…", "status": "accepted¦applied¦rejected", "reason": "…"}

- Send accepted when you take the command, then applied once guidance has started (Find) or stopped (Stop).

- Or send rejected with a reason, such as session_mismatch, expired or position_unavailable.

- The server ignores acks for unknown, other-session or already-finished commands, and never replies to them.

## 3.3 Registration: `POST /v1/items` (multipart `photo` + `name`), from the app's setup flow or from Photon

- Photo: any normal upright photo; EXIF is fine here, and the server applies it. Send JPEG (convert HEIC on the phone).

- `source` (optional form field): who registers it, app (the default: the app's setup flow) or photon (a caretaker). It lets one source's items be removed together (§5).

- Reply: {item_id, quality: "ok"|"retake", reason}. On retake, say "I can't see the {name} clearly, try again."

- `GET /v1/items` lists every item, with these fields (labels made before an app launch come from here):

  - item_id, name, position, surface, state;

  - last_seen_wall;

  - label_text, label_audio_url, size_m;

  - source: app, photon, or seed (the stub's built-in fake items).

- Saved across server restarts: every registration and label, with its photo (or touch crop) and voice note, under MARCO_DATA/registry/.

  - On start, the server reloads them and registers the photos with the detector again, so detection works at once.

  - Positions are not saved: an item is placed again when the camera next sees it. Until then it reads state: "registered".

## 3.4 ARWorldMap rules (Utkarsh), so positions survive relaunch

- Save getCurrentWorldMap about every 30 s and when the app goes to the background.

- On launch, load it as initialWorldMap. Wait until tracking is normal after relocalizing, then send map: relocalized and resume frames.

- If relocalizing hasn't worked after 30 s of walking:

  - start a fresh map;

  - send map: new_map;

  - say "Getting my bearings." (Marco never says its own wake word: the mic would hear it.)

The server keeps the items but marks their positions stale until they're seen again.

## 3.5 Spoken replies: `POST /v1/speech` (JSON)

The phone's existing Lines and guidance logic decide what to say. This route

converts that exact text to speech; it does not generate an answer or recognize

speech. It uses the same fail-closed Tailscale authentication as every other route.

{"text": "Polo! On your left, two steps, on the table."}

- text: 1–360 characters; whitespace is trimmed. Blank text and NUL are rejected (422).

- 200: Content-Type: audio/wav, mono 24 kHz signed 16-bit PCM in a WAV

container, at most 20 seconds. The server uses the standard 44-byte PCM RIFF

header followed by sample data; the phone validates format and lengths before

caching. Cache-Control: private, no-store prevents HTTP

caching; X-Marco-Speech-Provider: elevenlabs identifies the provider and

X-Marco-Speech-Cache: hit|miss identifies reuse of a cached/pending request.

- 503: unconfigured key, provider failure, invalid audio, saturation or

timeout. The phone falls back to Apple's AVSpeechSynthesizer. The server's

deadline is 2.5 s and the phone's is 3 s; after failure the phone uses Apple for

uncached lines for 15 s before trying again. 403: caller not authorized.

- Server cache: at most 128 entries / 8 MiB, keyed by voice, model and text;

duplicate pending lines share generation. Phone cache: 64 entries / 8 MiB,

cleared when the configured server changes. Caches are memory-only.

- A newer line, wake blip, Stop or stopped session cancels pending playback.

Late replies cannot speak an old instruction. The phone converts the WAV to

its existing mono spatial voice node; world anchors, beeps, head fusion and

the 60 Hz guidance loop remain on the phone. No TTS request per sensor tick.

- Key and stock voice/model configuration stay on the server. Defaults:

ELEVENLABS_VOICE_ID=JBFqnCBsd6RMkjVDRZzb,

ELEVENLABS_TTS_MODEL=eleven_flash_v2_5. See server/.env.example.

# 4. Server to phone (Vedant sends over `/v1/phone`)

{"type": "answer", "query_id": "…", "kind": "find", "found": true, "item_id": "…", "name": "keys", "position": [x, y, z], "surface": "table", "seen_ago_s": 600, "confidence": 0.82, "state": "present"} {"type": "answer", "query_id": "…", "kind": "find", "found": false, "reason": "not_registered", "name": "cat"} {"type": "answer", "query_id": "…", "kind": "find", "found": false, "reason": "never_seen", "item_id": "…", "name": "wallet"} {"type": "answer", "query_id": "…", "kind": "find", "found": false, "reason": "not_where_last_seen", "item_id": "…", "name": "mug", "last_position": [x, y, z], "surface": "shelf"} {"type": "answer", "query_id": "…", "kind": "what_is_this", "status": "known¦unknown¦no_touch", "item_id": "…", "name": "cumin", "label_text": "That's the cumin, love", "label_audio_url": "/v1/audio/…", "touch_id": "…"} {"type": "answer", "query_id": "…", "kind": "set_place", "name": "door", "ok": true} {"type": "touch", "touch_id": "…", "item_id": "…" , "name": "cumin", "position": [x, y, z], "t_device": 81235.1} {"type": "touch_end", "touch_id": "…"} {"type": "hand_proximity", "item_id": "…", "distance_m": 0.27} {"type": "event", "kind": "seen¦put_down¦moved¦picked_up¦missing¦left_behind¦label_added", "item_id": "…", "name": "mug", "position": [x, y, z], "from_position": [x, y, z], "surface": "shelf", "t_wall": "…"} {"type": "label_ready", "item_id": "…", "name": "cumin", "label_text": "…", "label_audio_url": "/v1/audio/…"}

- seen_ago_s may be null.

- touch.item_id is null for an unknown thing being touched.

- hand_proximity is sent 15 times a second while guiding: the hand's distance to the target, which drives the reach pings and the steady tone in §7. It's measured from the fingertip, or from the knuckles or wrist when the back of the hand hides the fingertip from the chest camera.

- Touching a known item while not being guided to it names it: the phone says its label text, or "That's the {name}," through §3.5 with Apple fallback.

- label_audio_url and GET /v1/audio/{id} remain for stored-data and older-client compatibility. The current iOS app does not download or prefer caretaker recordings; it caches synthesized replies instead.

Dashboard command (Find or Stop pressed on the dashboard, §6), sent only to the socket bound to the current session:

{"type": "command", "command_id": "…", "session_id": "…", "action": "find¦stop", "item_id": "…", "name": "keys", "position": [x, y, z], "surface": "table", "expires_wall": "2026-10-17T22:00:10.000Z"}

- Ignore and reject:

  - drop a repeated command_id;

  - reject a command for another session_id;

  - reject one that arrives after expires_wall (with about 2 s of clock slack): never start it late.

- Find: guide to exactly item_id at position, the same way as a voice Find; don't look the item up by name. Reject with position_unavailable when tracking isn't usable.

- Stop: stop guidance only; the camera and streams stay on.

- Ack: send command_ack (§3.2) for every command you act on or refuse.

# 5. Photon to server (Utkarsh's Photon service)

- Connect through the `tailscale serve` URL, even on the server Mac, so the allow-list sees the bridge's tailnet login.

- Requests: listen on WebSocket /v1/photon for {"type": "label_request", "touch_id": "…", "crop_url": "/v1/touch/…/crop.jpg", "question": "What is this?"}. These are sent when the user asks "what is this?" about an unknown item.

- Reconnect delivery: the server retains up to 128 unanswered requests for 10 minutes and replays them to an authenticated bridge when it connects. Delivery is at least once: deduplicate by touch_id, including across bridge restarts. Saving a label resolves that request. This buffer and touch crops are memory-only; a server restart loses them. The bridge can use its saved crop if a later answer encounters an expired touch (404).

- Asking: send the crop (the touched item boxed) to the caretaker over iMessage.

- Replying: post the answer to POST /v1/labels as multipart:

  - touch_id, name (short, e.g. "cumin"), and text (the caretaker's words, which the phone speaks);

  - no audio is sent by the current bridge. The optional legacy upload field remains compatible with older clients and saved data.

The server:

  - registers the item from the crop;

  - anchors it where it was touched;

  - sends label_ready to the phone;

  - replies {"status": "ok", "item_id": "…"}.

- Retried answers: posting the same retained touch_id again reuses its existing item_id, so a lost HTTP response does not create a second item. It can emit label_ready again. Prefer item_id for subsequent corrections. This does not make arbitrary POST /v1/items uploads idempotent.

- Delivery status: the phone's unknown answer means a helper request was generated, not that iMessage accepted or delivered a photo. This contract has no provider-delivery acknowledgement to the phone.

- Corrections: post again with that item_id (instead of touch_id), the corrected name and typed text. A standalone recording gets a request to type the name; it is not downloaded, transcribed or uploaded as a label.

- Caretaker-initiated setup: the caretaker sends a photo with a caption. Post it to POST /v1/items with name = the caption.

- `source=photon` goes on every Photon POST /v1/items and POST /v1/labels (an item a label creates gets this source).

- Starting over (testing): when a caretaker texts /restart, the bridge calls DELETE /v1/items?source=photon.

  - The server forgets every Photon item, with its label, voice note and saved files.

  - The app's own setup items (source=app) are untouched.

  - The reply is {"status": "ok", "removed": [{"item_id": "…", "name": "…"}]}.

  - source is required (422 without it), so nothing is ever removed by accident. The route sits behind the same auth gate.

# 6. Server to dashboard

The dashboard's API (server/marco/dashboard.py). Its payload is frontend/src/lib/schema.ts, exported as frontend/docs/dashboard.snapshot.schema.json; the behaviour is in frontend/docs/fastapi-handoff.md. Every route also answers under /api/…, the browser's base path, and every route is behind the auth gate.

| Route | What it does |
| --- | --- |
| GET /dashboard/snapshot | one complete dashboard.snapshot envelope: schemaVersion 1, streamId, a strictly increasing sequence shared with /events, and sentAt |
| WS /events | a snapshot at once, then after each change (at most 5 per second) and at least once a second; in between, dashboard.sound messages as they happen (below) |
| POST /dashboard/commands | Find ({commandId, deviceId, type: "find", itemId}) or Stop ({commandId, deviceId, type: "stop"}) |
| GET /media/frames/<token>.jpg, /media/items/<item_id>.jpg, /media/audio/<id>, /media/touch/<touch_id>.jpg | the recent upright frames (the last 30), item photos, label audio and touch crops |
| GET /app/… | the built dashboard, when MARCO_DASHBOARD_DIR points at frontend/dist (one origin with the API) |

- `deviceId` is the phone's current session_id.

- Commands: they go to the bound phone as command (§4) and come back as command_ack (§3.2).

  - Success: HTTP 202 {commandId, status: "pending"} means only that the command was sent.

  - Status: the snapshot's lastCommand follows the phone: accepted, then completed (guidance started or stopped) or rejected with a reason.

  - No answer: after 10 s the command is rejected with no_answer, which means the outcome is unknown.

  - Retries: a retry with the same commandId and body sends nothing again; reusing an ID for a different command gets 409.

  - Errors: 400 invalid_request; 404 unknown_item; 409 phone_disconnected, session_mismatch, busy, location_unavailable or command_conflict, each with a body {"error": {"code", "message"}}.

- Mapping:

  - Items: the core's world items; lastSeen.position is null when the item's position is stale (a new map).

  - Wearer: the latest pose in the current map.

  - Navigation: the phone's guidance telemetry:

    - behind, turning, facing or approach become guiding;

    - reach becomes within_reach;

    - touched becomes found;

    - after 3 s of silence it's idle (or unavailable if the phone is gone).

  - Devices: the phone is connected while a message arrived in the last 5 s. AirPods are connected while poses are AirPods-fused, and unknown otherwise.

  - Registrations: label_request and label_ready, plus POST /v1/items results.

  - recentMemory: seen, moved/put_down and label_added timeline events.

  - Room layout: the latest room_layout for the current map (below). map.walls holds its walls as {from: {x, z}, to: {x, z}}. map.landmarks holds its furniture as {id, kind, x, z, width, depth, yawDeg, shape}:

    - x, z is the centre; width runs across, depth front to back;

    - yawDeg is the way the front faces, in the wearer's convention (0 = world -z, 90 = +x);

    - shape is round or rect;

    - kinds today are table, chair, sofa, bench, bed, storage, tree and plant; draw unknown kinds as a plain block.

The layout is display only. It's LiDAR geometry, so it can be wrong (a doorway the scan hasn't seen through yet can show as a wall). It's dropped when the map changes. Without a layout, walls is null and landmarks is absent.

- Room name: MARCO_ROOM_ID and MARCO_ROOM_NAME.

- What the wearer hears: the phone's sound and sound_field (§3.2) go out on /events at once, between snapshots, not throttled:

```json

{"type": "dashboard.sound", "schemaVersion": 1, "streamId": "…", "sentAt": "…",

"sound": {"kind": "cue", "tDevice": 81235.2, "cue": "beep", "source": "beacon", "anchor": "item",

"azimuthDeg": -150.0, "elevationDeg": -12.0, "distanceM": 1.6, "muffled": true,

"text": null, "durationS": null, "audioUrl": null}}

{"type": "dashboard.sound", "schemaVersion": 1, "streamId": "…", "sentAt": "…",

"sound": {"kind": "field", "tDevice": 81235.3, "headYawDeg": 12.3,

"beacon": {"azimuthDeg": -150.0, "elevationDeg": -12.0, "distanceM": 1.6, "muffled": true}, "voice": null}}

`

  - Only the bound phone session's are sent. A malformed one is dropped.

  - audioUrl is a voice memo's /media/audio/<id>, when there is one. Field sources carry anchor (voice) or muffled (beacon).

  - A new /events connection gets the latest field at once if it is under 1 s old.

  - Dashboards that don't know dashboard.sound ignore it.

  - `navigation.spokenInstruction` in the snapshot is the phone's latest say line, while it plays and for 3 s after.

  - The dashboard (frontend/) draws these in every view: the room map, the camera and the guidance panel. The schema is frontend/docs/dashboard.sound.schema.json.

The raw feed, WebSocket /v1/dashboard (read-only), which the adapter itself is built from:

| Message | Rate | Content |
| --- | --- | --- |
| frame | every frame (about 1/s) | upright JPEG for display, plus boxes in its pixels: [{item_id, name, box, score, present}] |
| world | 2/s | items [{item_id, name, position, surface, last_seen_wall, state}], user {position, heading_deg}, trajectory of the last 5 min |
| map_points | every 5 s | new voxel points [[x, y, z, r, g, b], …] (2–5 cm voxels) |
| room_layout | every 3 frames (MARCO_LAYOUT_EVERY, 0 = off), once the floor is known | {map_id, walls: [[[x, z], [x, z]], …], objects: [{kind, x, z, width, depth, yaw, height, shape}]} from server/marco/world/layout.py: walls and furniture shapes summarised from the LiDAR map |
| guidance | 10/s | the phone's guidance state (§3.2), plus target and user positions, for the radar |
| timeline | on change | events and sightings: what, when, where |

Also GET /v1/timeline?item_id=… and GET /v1/items. For debugging, GET /v1/debug/detection gives, for the last 30 frames, each item's presence scores and where it was placed (or why not); see server/README.md. The pre-recorded outside-view 3D video can be rendered from map_points plus world.

# 7. The spatial-audio sequence: who computes what

The phone runs this loop at 60 Hz for low latency (ios/Marco/Core/Guidance.swift). The server supplies the target and the touch.

- "Marco, where are my keys?" The phone sends query find.

- The answer arrives. The phone speaks "Polo! Behind you, two steps, on the table," spatialised at position:

  - the direction is relative to the body (chest heading), because "behind you" means behind your body;

  - clock position = round(relative azimuth / 30°), where 12 o'clock means ahead; 12, 3, 6 and 9 are said as "straight ahead", "on your right", "behind you" and "on your left";

  - steps = round(horizontal distance / 0.7 m), at least one;

  - within 0.6 m it says "Polo! Within reach, …" instead.

- Beacon from the item, after the opening line, rendered relative to the head: the fused yaw, advanced by the audio output latency so a sound is placed where the head will be when it is heard. Three sounds say where the item is without relying on the HRTF:

  - behind (|relative azimuth| above 110°, back under 100°): a low, muffled beep every 0.55 s;

  - off to one side: a plain beep every 0.4 s;

  - on the beam (step 4): a bright pulse whose pace is the distance (step 5).

The beacon is never played behind the head (BeaconWarp): errors up to 30° are doubled, 30-90° are squeezed into 60-90°, and anything further round is pinned at 90° on the shorter turning side. Once within reach, the direction is true.

- On the beam (|relative azimuth| within max(12°, atan(0.3 m / distance)), a corridor 30 cm either side of the item): the ordinary beacon switches to bright pulses without a confirmation dwell; the beacon lock releases 8° outside that, and drifting off swaps to the plain beep at once. The special alignment confirmation is separate:

  - hold inside the beam for 0.35 s to get one double ding and haptic;

  - rearm only after remaining more than 15° outside the beam boundary for 0.7 s (27° from the target far away, wider up close), then returning for another 0.35 s dwell;

  - leave at least 3 s between confirmation dings; holding alignment after cooldown alone does not repeat one;

  - coordinate updates for the same target preserve the latch and cooldown; a new Find resets them;

  - gaps longer than 0.2 s restart pending dwell timers. Confirmation pauses during the opening speech, arrival and hand guidance; these stages do not count toward dwell. Arrival and reach keep their own cues. Spatial placement and ordinary beeps still use the current angle every tick.

- Walking: the gap between bright pulses = clamp(0.3 × distance^0.8, 0.2, 0.95) s. Stop at 0.6 m, called earlier by 0.5 s of walking speed (at most 0.3 m): three rising notes and a haptic, then a soft ping from the item every 1.1 s. Walking back beyond 1.1 m re-arms it. Nothing is spoken between the opening line and the touch: the beeps do the guiding.

- Reach once hand_proximity.distance_m is 0.45 or less (it releases above 0.55, or when proximity is older than 0.5 s): pings from the item that rise in pitch (six steps, D5 to D6) and speed up (0.3 s apart at 45 cm, 0.13 s at 10 cm). The steady tone, the top pitch, plays at 5 cm or less and releases above 9 cm.

- On `touch` for that item: silence, then the item's typed label or "Got it." through §3.5. The phone's guidance sequence does not wait for cloud speech.

If an event moved (or put_down) arrives for the target, retarget to its new position.

# 8. What each person needs from the others

Vedant needs from Utkarsh:

- The frame upload (§3.1).

- The pose stream and the hand stream (§3.2).

- The map messages (§3.4).

- A 5-minute recorded session saved to disk in exactly these formats (frames, poses, hands), so Vedant can build against real data before the app is finished.

  - The server records every session it receives under server/data/sessions/<session_id>/. Running the app against the server is enough.

  - Until then, a Stray Scanner recording converts with server/tools/stray_session.py.

Utkarsh needs from Vedant:

- A stub server that accepts everything above and returns fake answer / touch / event messages (MARCO_MODE=stub, the default).

- Then the real ones (MARCO_MODE=real).

- Typed labels and the speech route (§3.5). Legacy label audio URLs remain optional compatibility fields.

- label_request events for Photon.

The dashboard needs from Vedant:

- the /v1/dashboard feed with recorded or fake data first (server/tools/replay.py), then live.

Everyone: the server is private:

- it binds 127.0.0.1 only and is reached through tailscale serve;

- every route goes through the fail-closed auth gate (server/marco/auth.py), which admits only TAILSCALE_ALLOWED_LOGINS.

# 9. Integration order

- Hour 0–2: agree this file. Vedant ships the stub server; Utkarsh gets the ARKit session and frame upload working; the dashboard renders fake feeds.

- Then: the recorded 5-minute session from the vest. Vedant runs the real pipeline on it offline; the dashboard replays it.

- First live path: register keys, put them down, ask "where are my keys?", and follow the sound to them, end to end.

- Then:

  - touch and "what is this?" with Photon labeling;

  - moved and left-behind events;

  - relaunch persistence.

# 10. Open questions (decide together)

- Vision orientation: .up versus .right for the hand pose. Test in hour one.

- Where the server runs: decided (2026-10-17). The Vultr GPU box (deploy/vultr/, marco-gpu) is the primary server; a laptop at the venue is the fallback. Nothing in this contract changes: clients only change the base URL.

- Who names places ("entry table"): the server names them by height, or the user names them during setup.

- Photon and dashboard routes. photon/ and frontend/ were built against the earlier requirements and POC 26 paths. Status of each:

  - Photon: decided (2026-10-17). photon/ adopted §5 as written, on Photon 2.0, with no format changes. The phone never talks to photon/. Two optional additions are proposed in STATUS.md (label_request_seen, label_unknown); they are not part of this contract.

  - Dashboard: decided (2026-10-18). The server serves frontend/'s own contract (§6, marco/dashboard.py). The phone gained command and command_ack for Find and Stop.

# 11. Changes since v0.1

- Compression: DEFLATE, where raw DEFLATE (Apple's .zlib) and zlib are both accepted.

- `session_id` is on every phone message (hand, guidance, query and map too).

- Hand message: gains map_id. The server uses the latest frame's intrinsics, and the joint names are spelled out.

- New query set_place door, with its answer.

- Find answers:

  - reason adds not_registered;

  - not_where_last_seen carries last_position and surface;

  - found answers carry state.

- Event kinds: added seen, missing and label_added.

- `POST /v1/labels`: takes text, and item_id for a follow-up voice note; it returns item_id.

- `GET /v1/items` fields are listed.

- §7: directions are relative to the body and sound to the head; there is a "within reach" line, the facing lock releases past 20°, and the steady tone releases above 40 cm.

- "Getting my bearings." replaces "Marco is getting its bearings."

# 12. Changes since v0.2

- Registrations and labels persist across server restarts (§3.3), under MARCO_DATA/registry/.

- `source`:

  - optional on POST /v1/items (default app) and POST /v1/labels (default photon);

  - listed in GET /v1/items.

- New route DELETE /v1/items?source=…, which forgets one source's items. Photon's /restart command uses it (§5).

- Dashboard API (§6): GET /dashboard/snapshot, WS /events, POST /dashboard/commands, GET /media/… and /app/…, each also under /api/…, all behind the auth gate.

- New phone messages for dashboard Find and Stop: command (server to phone, §4) and command_ack (phone to server, §3.2).

# 13. Changes since v0.3

- Output-only ElevenLabs: authenticated POST /v1/speech (§3.5), short exact text to mono WAV, bounded caching/timeouts and Apple fallback. iOS plays it through the existing spatial voice node.

- §7 guidance sounds (phone only; no message format changed): a beam that widens to 30 cm either side up close, three beacon sounds, a beacon never played behind, distance pace across the whole room, a speed-compensated stop (notes and a haptic, no words), graded reach pings with the steady tone at contact (5 cm), and head yaw advanced by the audio latency. The only words during a find are the opening line and the touch.

- Stable facing confirmation (§7 step 4; phone only): 0.35 s entry dwell, a wider sustained-away rearm and a 3 s repeat limit prevent repeated dings from small movements. Ordinary beacon direction/cadence remain responsive, and same-item position updates preserve the confirmation latch.

- Typed Photon labels: no Scribe, transcription or new caregiver audio labels. Photos and typed names, retries, corrections and source attribution stay. Stored legacy media and optional wire fields remain compatible.

- A hand over the guided item no longer makes it `missing` (§7 step 7). Close up, the detector usually can't see the item under the hand. That used to mark the item missing after 3 frames, which took it out of touch matching, so the touch, and with it the end of the search, never came.

  - Frames where a hand is within 35 cm of an item don't count as misses.

  - An item that disappears after a hand was at it is picked_up, not missing.

- Room layout on the dashboard map (§6):

  - a new raw-feed message, room_layout;

  - in the snapshot, map.walls is filled, and there is a new optional map.landmarks;

  - a new map_id on a frame now starts a fresh room map (old-frame voxels used to stay).

- What the wearer hears: new phone messages sound and sound_field (§3.2), relayed to the dashboard as dashboard.sound on /events; navigation.spokenInstruction now carries the phone's spoken line (§6).